3GPP TSG-SA WG3 LI Meeting # 8	Tdoc S3LI01_131

Aspen, Colorado, USA. 30 October – 1 November 2001

| CR-Form-v4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 33.108 |  |  |  | CR | CRNum |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 0.1.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Modification of the GLICv0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Nokia corporation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 22 October 2001 |  |  |  |  |  |
| Category:  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | Rel-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | S3 LI has approved a new WI on the support of IPv6 network element addressing. HI2 already supports 20 octets of the IPv6 address. HI3 port supports IPv6 for FTP, but not for GLICv0. This contribution offers a way to resolve the problem in a future proof manner. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Dynamic CC header is proposed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | GLICv0 lacks a number of relevant parameters. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | Normative annex C.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  | Related Tdoc: ‘S3LI01_130 DP on GLICv0 modification‘ and ‘S3LI01_116_GLIC_Header’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

Annex C (normative):
UMTS HI3 Interface

There are two possible methods for delivery of content of communication to the LEMF standardized in this document:

-	LI Correlation Header over UDP/TCP

-	FTP

# C.1	LI Correlation Header

## C.1.1	Introduction

The header and the payload of the communication between the intercepted subscriber and the other party (later called: Information Element) is duplicated. A new header (later called: LIC-Header, see figure C.1) is added (see figure C.3) before it is sent to LEMF.

Data packets with the LIC header shall be sent to the LEA  via UDP/IP or TCP/IP.

## C.1.2	Definition of GLICv0 Header

GLIC header contains the following attributes:

- Correlation Number

- Message Type (a value of 255 is used for HI3-PDU’s).

- Direction

- Sequence Number

- Length

T-PDU contains the intercepted information.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| 1 |  | Version (‘0 0 0’) |  |  | ‘1’ | Spare ‘1 1‘ |  | DIR |  | ‘0’ |
| 2 |  | Message Type (value 255) |  |  |  |  |  |  |  |  |
| 3-4 |  | Length |  |  |  |  |  |  |  |  |
| 5-6 |  | Sequence Number |  |  |  |  |  |  |  |  |
| 7-8 |  | not used (value 0) |  |  |  |  |  |  |  |  |
| 9 |  | not used (value 255) |  |  |  |  |  |  |  |  |
| 10 |  | not used (value 255) |  |  |  |  |  |  |  |  |
| 11 |  | not used (value 255) |  |  |  |  |  |  |  |  |
| 12 |  | not used (value 255) |  |  |  |  |  |  |  |  |
| 13-20 |  | correlation number |  |  |  |  |  |  |  |  |

- Figure C.1: Outline of GLIC header

-	For interception tunneling  the GLIC header shall be used as follows:

-	Version shall be set to 0 to indicate the first version of  GLIC header.

-	DIR indicates the direction of the T-PDU:

"1" indicating uplink (from observed mobile user) and

"0" indicating downlink (to observed mobile user).

-	Message Type shall be set to 255 (the unique value that is used for T-PDU within GTP [12]).

-	Length shall be the length, in octets, of the signaling message excluding the GLIC header. Bit 8 of octet 3 is the most significant bit and bit 1 of octet 4 is the least significant bit of the length field.

-	Sequence Number is an increasing sequence number for tunneled T-PDUs. Bit 8 of octet 5 is the most significant bit and bit 1 of octet 6 is the least significant bit of the sequence number field.

-	Correlation Number consists of two parts:-	GGSN-ID identifies the GGSN which creates the Charging-ID

Charging-ID is defined in [12] and assigned uniquely to each PDP context activation on that GGSN (4 octets).

The correlation number consist of 8 octets and guarantees a unique identification of the tunnel to the LEA over a long time. The requirements for this identification are similar to that defined for charging in [12], chapter 5.4. Therefore it is proposed to use the Charging-ID,  defined in [12] , chapter 5.4 as part of correlation number. The Charging-ID is signaled to the new SGSN in case of SGSN-change so the tunnel identifier could be used “seamlessly” for the HI3 interface.

| 0 |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  | 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 |  |
| Charging –ID Octet 1 |  |  |  |  |  |  |  | Charging –ID Octet 2 |  |  |  |  |  |  |  | Charging –ID Octet 3 |  |  |  |  |  |  |  | Charging –ID Octet 4 |  |  |  |  |  |  |  | Octet 13-16 |
| GGSN-ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 17-20 |

Figure C.2: Outline of correlation number

The GLIC header is followed by a subsequent payload information element. Only one information element is allowed in a single signaling message.

|  | Bits |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| 1 – 20 | GLIC -Header |  |  |  |  |  |  |  |
| 21 –n | Information Element |  |  |  |  |  |  |  |

- Figure C.3: GLIC header followed by the subsequent payload Information Element

The Information Element contains the header and the payload of the communication between the intercepted subscriber and the other party.

## C.1.3	Definition of LICv1 message

LIC message is encoded/decoded in a dynamic manner. Figure C.4 outlines the dynamic LIC header.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| 1 |  | Version (‘0 0 1’) |  |  | ‘1’ | Spare ‘1 1‘ |  | DIR |  | ‘0’ |
| 2 |  | Message Type (value 255) |  |  |  |  |  |  |  |  |
| 3-4 |  | Length |  |  |  |  |  |  |  |  |
| 5-6 |  | Sequence Number |  |  |  |  |  |  |  |  |
| 7 |  | Type of the first IE |  |  |  |  |  |  |  |  |
| 8 |  | Length of the first IE |  |  |  |  |  |  |  |  |
| 9 – k |  | Value of the first IE |  |  |  |  |  |  |  |  |
| … |  | … |  |  |  |  |  |  |  |  |
| m |  | Type of the last IE (actual CC) |  |  |  |  |  |  |  |  |
| m+1 |  | Length of the last IE (actual CC) |  |  |  |  |  |  |  |  |
| (m+1) – n |  | Value of the last IE (actual CC) |  |  |  |  |  |  |  |  |

Figure 1: Possible outline of the dynamic LIC message and actual CC PDU

The following table lists possible IE type definitions.

Table 1. Possible IE type values in the dynamic LIC message

| IE name | IE Type value | Comments |
| --- | --- | --- |
| TimeStamp | 134 | Time Stamp. |
| Sequence Number | 141 | CC sequence number. |
| GPRSCorrelationNumber | 144 | LI Correlation Number identifier. |
| communication-Identity-Number | 145 | CIN identifier. |
| operator-Identifier | 146 | NOW/AP/SvP identifier. |
| Network-Element-Identifier | 147 | NEID identifier. |
| National-Parameters | 148 | National Parameters. |
| LIID | 254 | LIID. |

-	For interception tunneling  the LIC header shall be used as follows:

-	Version shall be set to 1 to indicate the first version of LIC message.

-	DIR indicates the direction of the T-PDU:

"1" indicating uplink (from observed mobile user) and

"0" indicating downlink (to observed mobile user).

-	Bit 8 of a field is the most significant bit and bit 1 is the least significant bit.

-	Message Type shall be set to 255 (the unique value that is used for T-PDU within GTP [12]).

-	Length shall be the length, in octets, of the following IE.

-	Sequence Number is an increasing sequence number for tunneled T-PDUs.

-	GPRSCorrelationNumber parameter is defined in the Annex B.4

In the LIC message a subsequent payload information element makes the last IE.

## C.1.4	Exceptional Procedure

With GLIC/LIC over UDP: the delivering node doesn’t take care about any problems at LEMF.

With GLIC/LIC over TCP: TCP tries to establish a connection to LEMF and resending (buffering in the sending node) of packets is also supported by TCP.

In both cases it might happen that call content gets lost (in case the LEMF or the transit network between MF and LEMF is down for a long time).

## C.1.5	Other Considerations

The use of IPsec for this interface is recommended. Ipsec is native protocol for the IPv6 suite.

The required functions in LEMF are:

-	Collecting and storing of the incoming packets inline.

-	Correlating of CC to IRI with the use of the correlation number in the GLIC header, or LIC message.

*** Next Modification ***