PROJECT:	T1P1 Activity on Lawful Intercept for UMTS

TITLE:	Recommended changes to TS 33.107 and TS 33.108 to address Issue #3, Roaming, of the issues list documented in T1P1/2002-004.

SOURCE:	Lou Degni

CALEA Implementation Section

14800 Conference Center Drive, Suite 300

Chantilly, VA 20151-3810

Tel:  	(703) 814-4729

Fax:  	(703) 814-4720

Sandra Lopez

Telcordia Technologies, Inc.

331 Newman Springs Road

Red Bank, NJ 07701

Tel:	(732) 758-3222

Fax:  	(732) 758-4196

e-mail:	slopez@telcordia.com

DATE:	March 26-28, 2002

LOCATION: 	Irving, Texas

ABSTRACT:	Proposed text to address Issue #3, Roaming, of the issues list documented in T1P1/2002-004.

DISTRIBUTION:	T1P1.SAH

NOTICE

©2002 Telcordia Technologies, Inc. The proposals in this submission have been formulated to assist committee T1P1.  This document is offered to the committee as a basis for discussion and is not binding on Telcordia Technologies. The requirements are subject to change in form and numerical value after more study. Telcordia Technologies specifically reserves the right to add to, or amend, the quantitative statements made herein.  Nothing contained herein shall be construed as conferring by implication, estoppel, or otherwise any license or right under any patent, whether or not the use of information herein necessarily employs an invention of any existing or later issued patent.

[PAGE INTENTIONALLY LEFT BLANK]

A. Introduction

This contribution proposes text to address shortcomings in TS 33.108 in the area of reporting roaming related events. This contribution addresses Issue # 3 of the issues list documented in T1P1/2002-004 and action items assigned to CIS/Telcordia during the T1P1.SAH conference calls held on January 8, 2002 and January 14, 2002.

This contribution supercedes contribution T1P1/2002-009.  Much of the material is the same. However, the major differences in the proposals includes a structural change to separate the GGSN impacts from the HLR impacts as well as some clarifications to the specific information to be reported by the 3G HLR.

B. Discussion

Issue:  TS 33.108 does not support the capability to report the new “Serving System” when the intercept subject roams. In addition, TS 33.108 does not explicitly describe that the implementation will continue to deliver content of communication to the law enforcement monitoring facility (LEMF) when the content of communication is still available at the home network.

Discussion: GPRS networks can support two basic roaming scenarios:

- Mobile Station (MS) connects via the Visiting Serving GPRS Support Node (VSGSN) and Home Gateway GPRS Support Node (HGGSN);

- MS connects via VSGSN and Visiting GGSN (VGGSN).

Precedent is established in J-STD-025 for reporting roaming events and continuing to deliver content of communication, when appropriate and available (e.g., SMS cases).

During discussion of a similar contribution to 3GPP, the issue regarding the need to support any of these additional requirements was raised based on the enactment of the Patriot Law.  A US participant stated that the Patriot Law extends the authorization of interception orders to multiple carriers and therefore there is no need to provide a notification of a change in serving system.  It has been further conjectured that the CALEA interception function would automatically cause interception to be invoked by the new serving system (e.g., new serving SGSN) after the user has roamed.

The following quote from the Patriot Law is relevant to the applicability to and extensibility of CALEA.

SEC. 222. ASSISTANCE TO LAW ENFORCEMENT AGENCIES.

Nothing in this Act shall impose any additional technical obligation

or requirement on a provider of a wire or electronic communication

service or other person to furnish facilities or technical assistance.

A provider of a wire or electronic communication service,

landlord, custodian, or other person who furnishes facilities or technical

assistance pursuant to section 216 shall be reasonably compensated

for such reasonable expenditures incurred in providing such facilities or assistance.

In essence, the Patriot Law allows an LEA to serve the same court order to multiple carriers to seek assistance rather than getting an order for each carrierCALEA requires the carrier to be able to report on the identity of the new carrier when the subject's call is handed-off (e.g. via the "serving system" message).  This requirement has not been removed, changed or altered in any way, not by the USA Patriot Act and not by any other law.

In addition, the network must be able to intercept and deliver content of communications to LEA when the content is routed via the home GGSN. The packet network architecture was designed in such a way that the GGSNs are provided with significant capabilities to serve the user (e.g., assignment of IP address, access to some subscription data for the user for the purpose of PDP context establishment).  In being able to serve these individual users, unlike the circuit switched tandem switch that handles voice calls, the GGSN is uniquely positioned to be able to intercept packets from interception subjects, whether these packets are related to roaming or not.  The non-roaming case is already clearly supported by TS 33.108.  Support of interception in the case of roaming is a minor extension of the GGSN interception capabilities of TS 33.108.

Requirement: This capability is required by the CALEA Law, which has the following provision for this capability:

“A telecommunications carrier offering a feature or service that allows subscribers to redirect, hand off, or assign their wire or electronic communications to another service area or another service provider or to utilize facilities in another service area or of another service provider shall ensure that, when the carrier that had been providing assistance for the interception of wire or electronic communications or access to call-identifying information pursuant to a court order or lawful authorization no longer has access to the content of such communications or call-identifying information within the service area in which interception has been occurring as a result of the subscriber's use of such a feature or service, information is made available to the government (before, during, or immediately after the transfer of such communications) identifying the provider of wire or electronic communication service that has acquired access to the communications.”

Law enforcement agencies need access to all available authorized content of communications and IRI, and any additional information that would assist law enforcement in determining the service area or other carrier(s) that have access to any information. Law enforcement agencies (LEAs) need:

- access to the intercept subject’s communications until the carrier’s network no longer has access to the communication.

- Network Operators (NOWs)/ Access Provider (APs)/ Service Providers (SvPs) to provide as part of Intercept Related Information (IRI) the identity of the new NOW/AP/SvP and/or service area to law enforcement when access to an intercept subject’s communications cannot be maintained. The identity of the new NOW/AP/SvP and/or service area should be provided to law enforcement as soon as it is available.

- to be provided with any information that will permit the LEA to determine or infer the new NOW/AP/SvP’s or service area’s identity if this information is unavailable.

Recommendation: The author recommends the addition of new triggers to report the roaming event and related information.  Changes must be made o TS 33.107 and TS 33.108.

- Proposed Changes to TS 33.107 to address GGSN impacts of Roaming

  - Changes to Clause 7.2.1 of TS 33.107

Add the following as the new last paragraph for this Clause:

In the case where the 3G GGSN is performing interception of the content of communications, the interception subject roams (within the same national jurisdiction as the home network), and the 3G GGSN continues to be involved in the handling of the content of communications, the 3G GGSN shall continue to perform the interception of the content of communications.

- Proposed Changes to TS 33.108 to address GGSN impacts of Roaming

  - Add a Clause to address provision of IRI at 3G GGSN when roaming

Add Clause 6.6 to address reporting IRI when intercepting at the 3G GGSN for roaming scenarios.

## 6.6	IRI for packet domain at 3G GGSN

When the interception subject is authorized for service with another NWO/AP/SvP or in another service area and interception is occurring at the Home GGSN, the Home GGSN is capable of continuing to perform limited surveillance on the interception subject ongoing communications.

The Home GGSN shall report IRI for the following events when the interception subject is authorized for service with another NWO/AP/SvP or in another service area and the interception subject’s communications continue to be routed through the Home GGSN:

-	PDP context activation;

-	PDP context deactivation;

-	Start of interception with PDP context active;

-	Packet Activity.

  - Add a Clause to address Provision of Content of Communication When 3G GGSN Interception is active and Intercept Subject is Roaming

Add Clause 6.7 to address Content of Communication Interception at the 3G GGSN for roaming scenarios.

## 6.7	Content of Communication for packet domain

When the interception subject is authorized for service with another NWO/AP/SvP or in another service area and interception is occurring at the Home GGSN, the Home GGSN is capable of continuing to perform surveillance on the interception subject ongoing communications.

The Home GGSN shall deliver content of communication (if LEA is authorized to receive such information) when the interception subject is authorized for service with another NWO/AP/SvP or in another service area (within the same national jurisdiction as the home network), and the interception subject’s communications continue to be routed through the Home GGSN.

- Proposed Changes to TS 33.107 to address HLR Impacts of Roaming

  - Changes to Clause 7.3 of TS 33.107

Change the first two paragraphs as follows:

Intercept Related Information (Events) are necessary at the Mobile Station Attach, Mobile Station Detach, PDP Context Activation, Start of intercept with PDP context active, PDP Context Deactivation, RA update, Roaming, and SMS events.

Figure 21 shows the transfer of intercept related information to the DF2. If an event for / from a mobile subscriber occurs, the 3G GSN or the 3G Home Location Register (HLR) sends the relevant data to the DF2.

Also, the diagram for FIGURE 21 must be modified to include the (HLR) as a source of IRI.

Figure 21: Provision of Intercept Related Information

  - Changes to Clause 7.3.1 of TS 33.107

The following information needs to be transferred from the 3G GSN or the 3G HLR to the DF2 in order to allow a DF2 to perform its functionality:

  - Changes to Clause 7.3.2 of TS 33.107

Change the first paragraph as follows:

There are seven eight different events in which the information is sent to the DF2 if this is required. Details are described in the following section. The events for interception are configurable (if they are sent to DF2) in the 3G GSN or the HLR and can be suppressed in the DF2.

Add the following text:

The following events are applicable to the HLR:

- Roaming.

Change the paragraph immediately preceding Table 2 as follows:

A set of fields as shown below is used to generate the events. The events transmit the information from 3G GSN or 3G HLR to DF2. This set of fields as shown below can be extended in the 3G GSN or 3G HLR, if this is necessary as a national option. DF2 can extend this information if this is necessary as a national option e.g. a unique number for each surveillance warrant.

  - Changes to Table 2 of TS 33.107

Event Type Description which type of event is delivered: MS attach, MS detach, PDP context activation, Start of intercept with PDP context active, PDP context deactivation, SMS, Cell and/or RA update, Roaming.

Event Date Date of the event generation in the 3G GSN or the 3G HLR.

Event Time Time of the event generation in the 3G GSN or the 3G HLR.

New Network Identifier Identity of the new NWO/AP/SvP serving the intercept subject.

New Serving System Identity of the new serving system serving the intercept subject.

Roaming Address Address used to identify the intercept subject while roaming.

  - Addition of Clause 7.4.10 to TS 33.107

- 7.4.10	Roaming

The Roaming report event is generated at the 3G HLR, when the 3G HLR has detected that the intercept subject has roamed.  The fields will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party (if applicable)

Event Type

Event Time

Event Date

Access Point Name

PDP Type (if applicable)

New Network identifier (If applicable)

New Serving System (If applicable)

Roaming Address

IAs (if applicable)

- Proposed Changes to TS 33.108 to address Roaming

  - Changes to Clause 6.5 of TS 33.108

The following text changes need to be incorporated into Clause 6.5 of TS 33.108:

There are eight nine different event types received at DF2 level. According to each event, a Record is sent to the LEMF if this is required. The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF.

Table 1: Mapping between UMTS Data Events and HI2 records type

| Event | IRI Record Type |
| --- | --- |
| GPRS attach | REPORT |
| GPRS detach | REPORT |
| PDP context activation (successful) | BEGIN |
| PDP context activation (unsuccessful) | REPORT |
| Start of intercept with PDP context active | BEGIN |
| PDP context deactivation | END |
| Cell and /or RA update | REPORT if no PDP context is active CONTINUE if, at least, one PDP context is active |
| SMS | REPORT |
| Roaming | REPORT |

  - Additions to Table 2 of TS 33.108

The following additional parameters need to be added to Table 2 of TS 33.108:

Table 2: Mapping between Events information and IRI information

| parameter | description | HI2 ASN.1 parameter |
| --- | --- | --- |
| New Network Identifier | Reports the new NWO/AP/SvP identity. | networkIdentifier |
| New Serving System | Reports new serving system information. | ServingSystem-Information |
| Roaming Address | This field provides the address associated with the interception subject while roaming. | roamingAddress |

  - Changes to Clause 6.5.1.1: REPORT record information

Add a new IRI triggering event for the REPORT record to be sent to report information about the NWO/AP/SvP providing service to the interception subject with terminal mobility, when the terminal is authorized for service with another NWO/AP/SvP or in another service area. A new bullet needs to be added as shown below:

- a mobile station is authorized for service with another NWO/AP/SvP or in another service area.  The REPORT record shall be generated by the 3G HLR.

Table 6-8: Roaming REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed IMEI |  |  |
| observed PDP address | C | Provide, if available, to identify the PDP address of the intercept subject. |
| event type | C | Provide Roaming event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| PDP type | C | Provide, if available, to describe the PDP type of the observed PDP address. |
| new network identifier | C | Provide to identify the new NOW/AP/SvP. |
| new serving system | C | Provide to identify the new serving system. |
| lawful intercept identifier | M | Shall be provided. |
| roaming address | C | Provide to identify the address of the interception subject while roaming. |

  - Additions to Clause B.4 of TS 33.108

The ASN.1 encoding in Clause B.4 of TS 33.108 must be modified to define the new parameters required to be reported with the Roaming event.

newNetworkIdentifier		[31?] Network-Identifier OPTIONAL,

newServingSystem			[32?] ServingSystem-Information OPTIONAL,

roamingAddress			[33?] VisibleString (SIZE (1..32)) OPTIONAL,

-- address of target subscriber known to the HLR when the roaming

-- event is detected.

The following parameter definition needs to be added to the ASN.1 encoding in Clause B.4 of TS 33.108 for reporting the raoming events.

PARAMETER FORMATS

ServingSystem-Information ::= SEQUENCE

{

sgsnAddress [0] DataNodeAddress OPTIONAL,

ggsnAddress [1] DataNodeAddress OPTIONAL,

mscAddress  [2] DataNodeAddress OPTIONAL,

-- Must include at least one.

…

}

The ASN.1 encoding of the GPRSEvent parameter in TS 33.108 needs to be modified to add a codepoint for this event (See Clause B.2.6). Within the ASN.1, the following value should be added

GPRSEvent ::= ENUMERATED

{

pDPContextActivation 					(1),

startOfInterceptionWithPDPContextActive	(2),

pDPContextDeactivation					(4),

gPRSAttach 								(5),

gPRSDetach 								(6),

locationInfoUpdate 						(10),

sMS 									(11),

roaming									(13?),

...

}

-- see ref [10]

Roaming is defined as the ability for a cellular customer to automatically make & receive voice calls, send & receive data, or access other services when traveling outside the geographic coverage area of the home network, by means of using a visited network. Roaming is technically supported by mobility management, authentication and billing procedures. Establishing roaming between network operators is based on - and the commercial terms are contained in - Roaming Agreements.